CAGAYANSTAT

COLLEGEOFENGINEERING

EUNIVERSITY	0

ANDARCHITECTURE

CARIGCAMPUS

# DEVELOPMENT OF A LOCALIZED REAL-TIME

# FREQUENCY MONITORING SYSTEM FOR STRUCTURAL

# HEALTH MONITORING USING MEMS IMUS AND ESP32

Presented to the faculty of

College of Engineering and Architecture

# CAGAYAN STATE UNIVERSITY

Carig Campus

Tuguegarao City, Cagayan

In Partial Fulfillment of the Requirements for the Degree

# BACHELOR OF SCIENCE IN ELECTRONICS ENGINEERING

JAN CLYDE T. TALOSIG

August 2026

Date of Submission

Table of Contents

Title Page ...…………………………………………………………………………………1

INTRODUCTION	4

1.1 Background of the Study	4

1.2 Conceptual Framework	6

1.3 Statements of the Problem	7

1.4 Objectives of the Study	7

1.5 Scope and Delimitation	8

1.6 Significance of the Study	8

1.7 Definition of Terms	9

CHAPTER 2	10

REVIEW OF RELATED LITERATURE	10

2.1 Hardware Components	11

2.1.1 ESP32 Microcontroller in IoT Systems	11

2.1.2 MEMS IMU Sensors for Vibration Monitoring	11

2.1.3 Signal Processing in SHM Systems	12

2.2 Software and Development Tools	12

2.2.1 Arduino IDE and Firmware Development	12

2.2.2 Web-Based Visualization Tools	13

2.2.3 AI-Assisted Development Tools	13

2.3 Statistical Tools	14

2.3.1 FFT and Frequency Analysis	14

2.3.2 Mean, Standard Deviation, and Signal Analysis	14

2.4 Other Related Studies	14

2.4.1 Structural Health Monitoring Systems	14

2.4.2 IoT-Based Monitoring Applications	15

2.4.3 Prototyping in Engineering Research	15

Synthesis of the Review	15

Chapter 3: Methodology	16

3.1 General Overview	16

3.1 Study Area (if applicable)	17

3.2 Research Design	17

3.3 Materials, Instruments, or Respondents	18

Hardware Components	18

3.4 Population and Sample	20

3.5 Research Instrument	20

3.6 Data Gathering Procedure	21

3.7 Reliability Test	22

3.8 Statistical Tools	23

3.9 Ethical Consideration	25

Chapter 4: Results and Discussion	26

4.1 System Functionality and IMU Performance Testing	26

Table 4.1.a — IMU Trial Data Summary (X, Y, Z Axes per Session)	29

Table 4.1.b — System Response Success Rate Summary	30

4.2 Resonance Frequency Extraction Validation	32

4.2.1 Overview	32

4.2.2 Extraction Performance Data	32

Table 4.2.a — Resonance Frequency Extraction Results per Session and Axis	33

4.2.3 Extraction Validation Summary	36

Table 4.2.b — Frequency Extraction Success Rate Summary	37

4.3 Real-Time Monitoring and Web Interface Evaluation	38

4.4 Overall Effectiveness Evaluation	41

Table 4.4 — Objectives vs. Outcomes Summary	43

CHAPTER V	46

SUMMARY, CONCLUSIONS, AND RECOMMENDATIONS	46

5.1 Summary of the Study	46

5.2 Conclusions	47

5.3 Recommendations	48

References	48

# INTRODUCTION

## 1.1 Background of the Study

Bridges are critical components of transportation infrastructure, serving as lifelines for economic activity, emergency response, and regional connectivity. Ensuring their safety and long-term performance is essential, particularly in the Philippines, which is highly exposed to natural hazards such as earthquakes, typhoons, flooding, and soil erosion. These environmental and operational stresses accelerate structural deterioration, making continuous monitoring increasingly important.

This need is not merely theoretical for Region 2. Within the span of a single year, Cagayan Valley experienced two major bridge failures that exposed both the structural risks of relying on periodic inspection and the steep economic cost of losing a river crossing. On February 27, 2025, a span of the newly opened Cabagan–Sta. Maria Bridge in Isabela — a ₱1.2-billion, 990-meter structure connecting Cabagan and Sta. Maria — collapsed after an overloaded dump truck crossed it, injuring six people and severing a key route across the Cagayan River for nearly a year. Beyond the loss of a billion-peso public asset only weeks after its completion, residents reported reduced farm and business income during the closure, as produce had to be rerouted and local commerce lost access to a primary trade corridor. DPWH's initial investigation attributed the failure to under-design, despite the bridge having been retrofitted only two years earlier.

Then, on October 6, 2025, the 45-year-old Piggatan Bridge in Alcala, Cagayan, gave way as multiple overloaded trucks carrying palay and corn crossed it during harvest season, injuring seven people and cutting off the route linking Alcala to Tuguegarao City and other Cagayan trading hubs. Local officials estimated that around 100,000 farmers in Alcala and neighboring towns depended on that crossing to move their produce, and with the detour stretching a ten-minute trip into an hour-long one, growers reported spoiled harvests and uncertain income even as they faced rising fuel and transport costs. Senate resolutions filed after both collapses explicitly cited significant economic losses alongside the casualties as justification for a nationwide inquiry into bridge safety, underscoring that structural failure in this region carries costs well beyond repair and reconstruction budgets — it disrupts entire agricultural supply chains and the livelihoods that depend on them.

These recent collapses have also raised concern over older structures still in daily use, chief among them the Buntun Bridge in Tuguegarao City. Opened in 1969 and stretching 1,369 meters across the Cagayan River, Buntun Bridge was for decades recognized as the longest river bridge in the Philippines — a title it held until the opening of the newer Camalaniugan Bridge in January 2026 — and it remains the sole direct connection between Tuguegarao City, Cagayan's provincial capital, and Solana, as well as the broader route toward Isabela, Kalinga, and Apayao. After 56 years of continuous service, the bridge has shown clear signs of aging: engineers have documented fatigue in its steel truss members, and holes and cracks have repeatedly appeared in its concrete deck, prompting emergency repairs as early as 2023. In March 2025, weeks after the Cabagan–Sta. Maria collapse, DPWH imposed a 15-ton load restriction on Buntun Bridge in response to worsening structural conditions, barring trucks, trailers, and fully loaded buses. The bridge has also been temporarily closed outright during major flood events, including during Typhoon Uwan in November 2025 and Typhoon Tino shortly after, when rising Cagayan River water levels forced authorities to shut it down for hours at a time. Because Buntun Bridge is the only crossing linking Tuguegarao City to Solana and the towns beyond, each closure or restriction disrupts commuting, emergency response, and the movement of goods for the entire provincial capital, with logistics and delivery operations across the region affected until the bridge reopens. Given its critical role, its documented deterioration, and the load and weather-based restrictions already being managed reactively rather than through continuous data, Buntun Bridge presents both an urgent monitoring need and a practical, accessible site for testing a real-time frequency-based SHM system under real operating conditions.

Together, these incidents illustrate how deterioration and overloading can go undetected between visual inspections, and how the consequences — injuries, severed regional connectivity, and cascading economic losses to farmers, businesses, and commuters — fall directly on communities in Region 2. They underscore the urgency of a monitoring approach that can track a structure's condition continuously rather than periodically, potentially flagging structural stress before it results in both human and economic loss.

Structural Health Monitoring (SHM) has emerged as a modern engineering approach for assessing the condition of structures through continuous or periodic measurement of structural responses. Among various SHM techniques, frequency-based monitoring is widely recognized as a reliable method for detecting structural changes. The natural and resonant frequencies of a structure are directly related to its mass and stiffness; therefore, any damage such as cracks, corrosion, or joint degradation can result in measurable shifts in frequency characteristics. In practice, vibration signals are acquired from the structure and processed to extract frequency information using techniques such as Fast Fourier Transform (FFT) and spectral analysis. These methods allow identification of dominant frequency components even in noisy environments, making them suitable for real-world applications.

Despite the effectiveness of frequency-based SHM, existing systems in the Philippines are largely dependent on imported technologies, including high-cost accelerometers, data acquisition systems, and proprietary software. These systems are often expensive and inaccessible for widespread deployment, limiting their use to large-scale infrastructure projects. As a result, many bridges — including those in Region 2 — rely primarily on periodic visual inspections, which, as the Cabagan–Sta. Maria, Piggatan, and Buntun Bridge cases demonstrate, may fail to detect early-stage structural deterioration or flag structures approaching their load limits before catastrophic failure occurs.

To address this gap, this study proposes the development of a localized, real-time frequency monitoring system for Structural Health Monitoring using MEMS-based Inertial Measurement Unit (IMU) sensors and an ESP32-S3 microcontroller. The system acquires vibration signals through MEMS IMUs, processes the data using FFT-based techniques, and extracts natural and resonant frequencies in real time. A web-based interface is also implemented for visualization and monitoring. The system is considered localized as it utilizes cost-efficient, locally accessible components and is designed to be adaptable to Philippine bridge conditions, and will be tested on Buntun Bridge in Tuguegarao City as a representative, high-criticality structure in Region 2. By focusing on affordability, portability, and real-time capability, the study aims to contribute to the development of practical SHM solutions for resource-limited environments — and to a monitoring approach that could have flagged the structural stress behind incidents like the Cabagan–Sta. Maria and Piggatan bridge collapses before they occurred.

## 1.2 Conceptual Framework

› Dominant Resonance Frequency (Hz)

› RMS Vibration Intensity

› Normalized Vibration Index

› CRIT / WARN / OK State Alerts

› Web Dashboard Visualization

› CSV Data Log (LittleFS)

› IMU Calibration & Offset Correction

› Moving Average Filter (MAF) Pre-Processing

› Hamming Windowing Function

› 1024-Point FFT Computation

› Peak Frequency Detection

› RMS Vibration Energy Calculation

› Bridge Structural Vibrations

› Dynamic Loads (Traffic, Wind, Environmental)

› Triaxial Acceleration Data (X, Y, Z)

› Raw IMU Signals from MPU-6050 Sensors

› Ambient Excitation Forces

Figure 1.1 Conceptual Framework Diagram

In this framework, structural vibrations and dynamic loads acting on bridge structures are treated as the independent variables, as they reflect the physical responses that indicate the condition of the structure. These vibrations may result from traffic loads, environmental forces, and material deterioration, which can lead to structural stress and potential damage if not detected early.

To address this issue, the study introduces a localized real-time Structural Health

Monitoring (SHM) system using MPU6050 IMU sensors and an ESP32 microcontroller.

The sensors capture vibration data, which are processed through calibration, filtering, Root Mean Square (RMS), and Fast Fourier Transform (FFT) to analyze vibration intensity and dominant frequencies. The processed data are then transmitted to a web-based visualization interface for real-time monitoring and anomaly detection.

## 1.3 Statements of the Problem

This study seeks to answer the following questions:

- How can a real-time frequency monitoring system be developed to acquire and process structural vibration data?

- How effectively can dominant structural frequencies be extracted from vibration signals under varying excitation and noise conditions?

- How can the system provide real-time monitoring and visualization of structural vibration characteristics?

## 1.4 Objectives of the Study

This study aims to develop a localized, real-time frequency monitoring system for bridge Structural Health Monitoring. Specifically, it aims to:

- Develop a real-time monitoring system capable of acquiring and processing structural vibration data.

- Extract and characterize dominant structural frequencies from acquired vibration signals using frequency-domain analysis under varying excitation conditions.

- Develop a real-time visualization interface for displaying structural vibration characteristics and frequency behavior.

## 1.5 Scope and Delimitation

This study focuses on the development and evaluation of a localized, frequency-based Structural Health Monitoring system for bridge structures. The system is designed to acquire vibration signals using MEMS-based IMU sensors, process the signals through FFT, and extract frequency characteristics for real-time monitoring. The hardware system utilizes an ESP32-S3 microcontroller for data acquisition and processing, and the extracted frequency data are transmitted to a web-based interface for real-time visualization.

The system will be tested on the Buntun Bridge in Tuguegarao City, Cagayan, as the representative structure for this study. Data collection is limited to this single bridge site; the system's performance and extracted frequency characteristics are specific to Buntun Bridge and are not claimed to generalize to bridges of different structural types, spans, or material conditions without further testing.

Since no prior instrumented baseline exists for Buntun Bridge, this study establishes its baseline frequency ranges using the developed device itself, based on vibration data gathered under varying vehicular traffic conditions. The resulting baseline is therefore internal to the system and this deployment, and is not validated against an external reference sensor or a separately verified ground truth.

The study is limited to frequency-based monitoring and does not include structural diagnosis, damage classification, predictive modeling, load-capacity analysis, or repair recommendations. Interpretation of any observed frequency shift as a specific damage type or severity level is outside the scope of this study.

The system is further limited by hardware constraints such as processing speed, memory capacity, and sensor resolution. Environmental factors such as temperature, humidity, and electromagnetic interference are not fully incorporated into the analysis. Data collection is also limited to the specific time periods and traffic conditions observed during testing, and may not capture the full range of seasonal, weather-related, or long-term structural variation.

## 1.6 Significance of the Study

This study contributes to the development of a cost-effective and accessible Structural Health Monitoring solution for bridge infrastructure in the Philippines. By utilizing MEMS-based IMU sensors and an ESP32-S3 microcontroller, the system offers a practical, locally deployable alternative to the expensive imported SHM technologies that have limited continuous monitoring to only a small number of large-scale infrastructure projects.

To residents and commuters in Tuguegarao City and Solana, particularly those who rely on Buntun Bridge as their only direct crossing, this study represents a step toward monitoring the condition of a structure the community depends on daily for travel, trade, and emergency access — reducing reliance on reactive measures such as load restrictions and emergency closures imposed only after visible damage or a critical event occurs.

For engineers and infrastructure inspectors, the system offers a supplementary tool for continuous, real-time observation of structural vibration behavior, providing data between scheduled visual inspections rather than relying on inspection intervals alone.

For DPWH and other government agencies responsible for bridge maintenance, the system supports more informed, data-driven decisions on monitoring priorities and maintenance timing, offering a low-cost option that could extend structural oversight to bridges — especially aging or heavily trafficked ones like Buntun Bridge — that would otherwise go unmonitored between inspections.

For the local economy and communities in Region 2, including farmers, traders, and businesses whose livelihoods depend on uninterrupted access across the Cagayan River, this study supports the broader goal of reducing the risk of unplanned bridge closures and the trade, transport, and income disruptions that follow, as seen in the Cabagan–Sta. Maria and Piggatan Bridge incidents.

For the academic and engineering research community, this study contributes a documented reference for the development of embedded and IoT-based SHM systems using locally accessible components, offering a foundation that future researchers can build on — whether by extending the system to other structures, incorporating additional sensing modalities, or advancing it toward damage classification and predictive capabilities.

Ultimately, the localized design of this study promotes technological independence in structural monitoring and demonstrates that meaningful SHM capability can be achieved using affordable, readily available components — encouraging further innovation in this space within resource-limited settings.

## 1.7 Definition of Terms

Baseline Frequency Range: The set of natural and resonant frequency values established for a specific structure under normal operating conditions, used as a reference point against which future vibration readings can be compared.

Excitation: An external force or disturbance — such as vehicular traffic, wind, or ambient vibration — that causes a structure to vibrate and produce a measurable dynamic response.

ESP32-S3: A microcontroller used for data acquisition, processing, and wireless communication in the system.

Fast Fourier Transform (FFT): A signal processing technique used to convert time-domain signals into frequency-domain data.

Frequency Monitoring: The process of analyzing vibration signals to extract frequency characteristics related to structural behavior.

Localized (System): In this study, refers to a system designed using cost-efficient, locally accessible components rather than imported, proprietary hardware, making it more feasible to deploy and maintain in resource-limited settings.

MEMS IMU (Micro-Electro-Mechanical Systems Inertial Measurement Unit): A sensor that measures acceleration and motion, used in this study for vibration data acquisition.

Natural Frequency: The inherent frequency at which a structure vibrates when disturbed, determined by its mass and stiffness.

Real-Time Monitoring: Continuous or near-instantaneous observation and processing of data.

Resonant Frequency: The frequency at which a structure experiences maximum vibration amplitude due to external excitation.

Structural Health Monitoring (SHM): A method of evaluating the condition of a structure through measurement and analysis of its response.

Vibration Signal: The raw time-domain data captured by the sensor, representing the structure's motion or acceleration over time, prior to frequency-domain processing.

# CHAPTER 2

# REVIEW OF RELATED LITERATURE

Structural Health Monitoring (SHM) has become a critical approach in ensuring the safety, reliability, and longevity of bridge infrastructure. Across various studies, SHM is consistently recognized as an essential tool for early detection of structural deterioration and damage. Sales et al. (2022) demonstrated that accelerometer-based monitoring systems are capable of detecting cracks in bridge structures through detailed vibration analysis, showing that even small changes in dynamic behavior can indicate structural issues. Similarly, Zanelli et al. (2021) developed a low-power wireless sensor node that was successfully validated in real-world conditions, proving that efficient data acquisition can be achieved while maintaining minimal energy consumption. Komarizadehasl et al. (2022) further emphasized that low-cost SHM systems are not only feasible but also practical for real-world bridge applications, especially in environments where budget constraints are a major concern. In addition, Rai et al. (2025) and Wang and Ke (2024) highlighted that SHM plays a crucial role in sustainable infrastructure management by enabling continuous monitoring and early identification of structural degradation, thereby reducing maintenance costs and preventing catastrophic failures. However, the retraction of Sivasuriyan et al. (2021) highlights the importance of methodological reliability and validation in SHM research, emphasizing that system accuracy and research integrity are critical in this field.

## 2.1 Hardware Components

### 2.1.1 ESP32 Microcontroller in IoT Systems

The ESP32 microcontroller has become a widely utilized platform in Internet of Things (IoT)-based monitoring systems due to its integrated Wi-Fi and Bluetooth capabilities, low power consumption, and efficient processing performance. In Structural Health Monitoring (SHM) applications, the ESP32 enables real-time data acquisition and wireless transmission, making it suitable for continuous monitoring of infrastructure.

According to El-Khozondar et al. (2024), ESP32-based systems are capable of reliably handling real-time sensing and communication tasks in monitoring applications. Similarly, Md Dzahir and Chia (2023) emphasized that power-saving features such as deep sleep mode significantly extend battery life, allowing long-term deployment in remote environments. Babiuch and Postulka (2021) further demonstrated that the ESP32 can efficiently manage multiple sensor inputs while maintaining stable performance. In addition, Froiz-Míguez et al. (2023) highlighted the microcontroller’s flexibility in supporting full-stack IoT systems, including firmware development and cloud-based integration.

These capabilities make the ESP32 an ideal choice for implementing low-cost and scalable SHM systems.

### 2.1.2 MEMS IMU Sensors for Vibration Monitoring

Micro-Electro-Mechanical Systems (MEMS) sensors play a crucial role in modern SHM systems due to their compact size, affordability, and high sensitivity. One of the most commonly used sensors is the MPU-6050, which combines a three-axis accelerometer and a three-axis gyroscope to capture both linear acceleration and angular velocity.

Patel and Singh (2025) highlighted that MEMS-based accelerometers are highly effective for continuous vibration monitoring due to their low power consumption and high responsiveness. Di Nuzzo et al. (2021) demonstrated the integration of MEMS sensors with IoT systems, enabling real-time data transmission and remote accessibility. Nguyen et al. (2018) further confirmed that low-cost MEMS-based systems can produce reliable results when proper calibration and signal processing techniques are applied.

The MPU-6050 sensor, in particular, allows multi-axis vibration analysis, which is essential in detecting structural anomalies such as cracks or instability. Its ability to generate detailed motion data makes it a valuable component in vibration-based SHM systems.

### 2.1.3 Signal Processing in SHM Systems

Signal processing is a fundamental component of SHM systems, as it transforms raw sensor data into meaningful information for structural analysis. Techniques such as Fast Fourier Transform (FFT) and Root Mean Square (RMS) are widely used to analyze vibration signals.

Zhang and Li (2023) explained that changes in natural frequencies are strong indicators of structural deterioration. Silva and Ribeiro (2022) further emphasized that frequency response analysis provides reliable insights into structural behavior under dynamic loads. Ramadhan et al. (2025) also demonstrated that combining computational models with vibration signal processing improves accuracy in structural evaluation.

These techniques enable SHM systems to detect anomalies by identifying deviations in vibration patterns, supporting early damage detection and preventive maintenance.

## 2.2 Software and Development Tools

### 2.2.1 Arduino IDE and Firmware Development

The Arduino IDE is commonly used for programming microcontrollers such as the ESP32. It provides a user-friendly interface for writing, compiling, and uploading firmware.

In SHM systems, the Arduino IDE facilitates the development of data acquisition routines, sensor integration, and communication protocols. Its extensive library support simplifies the implementation of sensor drivers and wireless communication features, making it suitable for rapid prototyping and embedded system development.

### 2.2.2 Web-Based Visualization Tools

Web-based visualization platforms play a significant role in SHM systems by providing real-time data monitoring and analysis. These tools allow users to access sensor data remotely through dashboards, enabling engineers to observe structural behavior and respond to anomalies efficiently.

Kim and Park (2021) emphasized that visualization systems improve decision-making by presenting data in an accessible and interpretable format. Real-time graphs, alerts, and historical data tracking enhance the usability of SHM systems and support continuous infrastructure monitoring.

### 2.2.3 AI-Assisted Development Tools

Artificial Intelligence (AI)-assisted tools have recently been integrated into system development to improve efficiency and accuracy. These tools assist in code generation, debugging, and optimization, reducing development time and minimizing errors.

Bagerzadeh Karimi (2026) demonstrated the use of AI in SHM systems for anomaly detection, highlighting its potential in improving system intelligence. AI-assisted development tools also support data analysis and pattern recognition, making them valuable in enhancing SHM system capabilities.

## 2.3 Statistical Tools

### 2.3.1 FFT and Frequency Analysis

Frequency analysis is a key method in SHM for identifying structural changes. The Fast Fourier Transform converts time-domain signals into frequency-domain representations, allowing the identification of dominant vibration frequencies.

This method is essential for detecting shifts in natural frequencies, which may indicate structural damage. Studies such as those by Zhang and Li (2023) confirm that frequency deviations are reliable indicators of deterioration.

### 2.3.2 Mean, Standard Deviation, and Signal Analysis

Statistical measures such as mean and standard deviation are used to analyze vibration signals and assess variability. These metrics help quantify the consistency and stability of structural responses.

The use of standard deviation allows researchers to detect abnormal variations in vibration data, while RMS values provide an estimate of signal energy. Together, these tools enhance the reliability of SHM systems in identifying structural anomalies.

## 2.4 Other Related Studies

### 2.4.1 Structural Health Monitoring Systems

Structural Health Monitoring (SHM) systems are widely recognized as essential tools for ensuring infrastructure safety. Sales et al. (2022) demonstrated that accelerometer-based systems can detect cracks through vibration analysis. Zanelli et al. (2021) validated a low-power wireless SHM system in real-world conditions, while Komarizadehasl et al. (2022) emphasized the practicality of low-cost SHM implementations.

Rai et al. (2025) and Wang and Ke (2024) further highlighted the importance of SHM in sustainable infrastructure management by enabling early detection of damage and reducing maintenance costs.

### 2.4.2 IoT-Based Monitoring Applications

IoT-based monitoring systems enhance SHM by enabling real-time data transmission and remote accessibility. Di Nuzzo et al. (2021) demonstrated the effectiveness of IoT-integrated MEMS sensors, while Kim and Park (2021) identified IoT as a key trend in modern monitoring systems.

These systems support automated alerts and continuous monitoring, improving response time and decision-making efficiency.

### 2.4.3 Prototyping in Engineering Research

Prototyping plays a vital role in engineering research, allowing researchers to test and validate system designs before full-scale implementation. Low-cost prototyping platforms such as the ESP32 and MEMS sensors enable rapid development and Ferreira et al. (2022) emphasized that scalable sensor deployment is essential for large infrastructure systems, highlighting the importance of flexible and cost-efficient prototypes in SHM research.

## Synthesis of the Review

The reviewed literature demonstrates that Structural Health Monitoring (SHM) has become an essential component in ensuring the safety and reliability of bridge infrastructure. Vibration-based monitoring techniques, particularly those utilizing accelerometers and signal processing methods such as FFT and RMS, are effective in detecting structural anomalies through changes in dynamic behavior.

The integration of MEMS sensors, such as the MPU-6050, has enabled the development of compact and cost-efficient monitoring systems. Additionally, IoT technologies, supported by microcontrollers like the ESP32, have enhanced SHM capabilities by enabling real-time data transmission, remote monitoring, and data visualization.

Despite these advancements, many SHM systems remain costly and complex, limiting their accessibility in resource-constrained environments. This highlights the need for affordable and efficient solutions.

In response, the present study proposes a low-cost, real-time SHM system that integrates MEMS-based sensing, ESP32 processing, and wireless monitoring. By combining established methodologies with accessible technologies, the study aims to contribute to the development of practical and scalable SHM solutions for real-world bridge monitoring applications.

# Chapter 3: Methodology

## 3.1 General Overview

This study employed a developmental and evaluative research approach to design, Implement, and assess a frequency-based Structural Health Monitoring (SHM) system for bridge applications. The development process followed an iterative engineering design cycle encompassing hardware Integration, firmware development, signal processing implementation, and system validation. The system leverages an ESP32-53 microcontroller coupled with multiple MEMS inertial measurement units (IMUs) to continuously acquire vibration data from a bridge structure. Acquired signals undergo onboard digital signal processing-specifically Fast Fourier Transform (FFT) with Hamming windowing to extract dominant resonance frequencies. Results are streamed to a real-time web-based dashboard for visualization and anomaly indication. The overall workflow proceeds from sensor data acquisition → signal conditioning frequency-domain analysis →→ visualization and evaluation.

## 3.1 Study Area (if applicable)

This study will be conducted at the Buntun Bridge,

located in Carig Sur, Tuguegarao City, Cagayan Valley, Philippines. The experimental setup will involve in-situ deployment on an actual bridge structure; instead of testing  using the developed Structural Health Monitoring

(SHM) system under controlled conditions.

Vibration data were acquired through the system’s sensors subjected to controlled and repeatable motion inputs to simulate varying signal conditions. Sensor placement and testing configurations were arranged to evaluate the system’s capability in capturing, processing, and analyzing vibration signals.

The controlled environment minimized external disturbances such as environmental noise and unpredictable loading conditions, allowing for accurate assessment of the system’s frequency extraction, signal processing performance, and anomaly detection capability. The laboratory setting was selected to ensure consistency, ease of testing, and reliable validation of the developed SHM system prior to potential field deployment.

## 3.2 Research Design

This study adopted a developmental-evaluative research design. The developmental component involved the systematic engineering of the SHM hardware and software system from requirements gathering through integration and deployment. The evaluative component Involved controlled testing and performance assessment of the developed system against defined metrics-namely, frequency extraction accuracy, real-time responsiveness, and anomaly detection capability.

The research followed a phased approach:

- Phase 1-System Design: Definition of hardware architecture, sensor configuration, and communication protocols.

- Phase 2-System Development: Firmware programming, FFT implementation, LittleFS data logging, and web dashboard development.

- Phase 3-Deployment and Data Collection: Installation of the system at the Bridge and acquisition of vibration data under ambient excitation conditions.

## 3.3 Materials, Instruments, or Respondents

The following hardware and software components were used in the development and deployment of the SHM system:

### Hardware Components

| Component | Specification/Role |
| --- | --- |
| ESP32-S3 Microcontroller | Dual-core processing unit; hosts FreeRTOS-based firmware, FFT computation, and web server |
| MPU-6050 IMU Sensors (×3) | 6-axis MEMS sensors for triaxial acceleration measurement; PC with ADO pin multiplexing |
| PCB (Custom-designed) | Integrates power regulation, sensor breakout headers, and communication interfaces |

Software Components

| Tool | Purpose |
| --- | --- |
| Arduino IDE/PlatformIO | Firmware development environment |
| FreeRTOS | Real-time dual-core task scheduling |
| arduinoFFT Library | 1024-point FFT computation with Hamming windowing |
| WebServer | Synchronous HTTP server for dashboard delivery |
| LittleFS | Onboard flash filesystem for CSV data logging |
| Three.js (WebGL) | 3D bridge heatmap visualization on the web dashboard |

## 3.4 Population and Sample

Since this study is instrumentation-based and does not involve human respondents, the concept of population and sample is applied in the context of data sampling and sensor coverage.

The population refers to the full continuum of vibration signals generated by the Bridge under real-world loading and ambient excitation conditions. The sample consists of discrete time-series vibration data collected at three (3) sensor nodes positioned at structurally significant points along the bridge. Each sensor node acquires acceleration data at a sampling rate of 167 Hz, yielding a frequency resolution suitable for detecting vibration modes in the range of 0-83.5 Hz (Nyquist limit). Data was collected over multiple measurement sessions to capture varied loading conditions (pedestrian traffic, vehicle transit, wind excitation).

## 3.5 Research Instrument

The primary research instrument of this study is the developed ESP32-S3-based SHM device itself, composed of three subsystems:

- Sensor Subsystem - Three MPU-6050 IMUs mounted at designated bridge nodes; each sensor streams raw 3-axis acceleration data via PC. The ADO pin multiplexing scheme allows sequential polling of all three sensors on a shared bus without address conflict.

- Processing Subsystem - The ESP32-S3 firmware applies a 1024-point FFT with Hamming windowing on acquired acceleration signals. The dominant frequency peaks extracted from the FFT output serve as the primary indicators of the bridge's modal characteristics. FreeRTOS dual-core scheduling separates data acquisition (Core 0) from FFT computation and server tasks (Core 1) to ensure deterministic sampling.

- Visualization Subsystem -A web-based dashboard served by the ESP32-S3's onboard WebServer renders live frequency spectra and a Three.js 3D heatmap of vibration intensity across sensor nodes. Color-coded thresholds visually flag frequency anomalies that may indicate

structural irregularities.

## 3.6 Data Gathering Procedure

Data collection proceeded through the following steps:

- Sensor Placement - The three MPU-6050 IMUs were mounted on designated

structural points of the Bridge using rigid mechanical fasteners to minimize mounting-induced noise.

- System Initialization - Upon power-up, the ESP32-53 firmware initializes the I'C bus, calibrates each IMU at rest, and establishes the web server for real-time monitoring access.

- Ambient Vibration Testing - Vibration data was collected under ambient excitation (no artificial force input), relying on natural traffic loads and environmental forces. This approach, consistent with Operational Modal Analysis (OMA) methods, allows extraction of natural frequencies without controlled excitation equipment.

- Data Logging - Raw and processed vibration data (time-domain acceleration and FFT frequency bins) were logged to the ESP32-S3's LittleFS flash storage in CSV format for offline post-processing and validation.

- Real-Time Monitoring - Simultaneously, the dashboard was monitored remotely via a connected device to observe live frequency spectra and heatmap behavior during data collection sessions.

- Multiple Sessions - Data collection was conducted across multiple time windows to capture varying load conditions and improve result representativeness.

## 3.7 Reliability Test

The reliability of the SHM system was evaluated through the following procedures:

Repeatability Testing - The system was subjected to repeated measurement sessions under similar amblent conditions to assess the consistency of extracted dominant frequencies. Standard deviation of peak frequency values across sessions was computed to quantify

repeatability.

Sensor Calibration Verification - Each MPU-6050 was calibrated prior to deployment by acquiring static baseline readings and computing offsets. Post-calibration residual noise floors were verified to fall within the manufacturer's specified range.

Noise Rejection Assessment - The effectiveness of the Hamming window function in suppressing spectral leakage was verified by comparing raw FFT outputs against windowed outputs across known test frequencies.

Cross-Sensor Consistency - Frequency outputs from the three sensor nodes were compared to identify consistent modal peaks, distinguishing genuine structural modes from localized noise artifacts.

## 3.8 Statistical Tools

The following statistical and signal-processing methods were applied in analyzing data generated by the SHM system:

| Tool/Method | Application |
| --- | --- |
| Fast Fourier Transform (FFT) | Conversion of time-domain acceleration signals to frequency-domain spectra; identification of dominant resonance frequencies |
| Hamming Window Function | Reduction of spectral leakage in FFT computation for cleaner frequency resolution |
| Peak Detection Algorithm | Identification of dominant frequency |
|  | peaks corresponding to structural vibration modes |
| Mean and Standard Deviation | Assessment of measurement repeatability across multiple data collection sessions |
| Signal-to-Noise Ratio (SNR) | Quantification of the quality of extracted frequency signals relative to background noise |
| Percentage Deviation | Comparison of system-detected frequencies against reference values or theoretical estimates for accuracy evaluation |

## 3.9 Ethical Consideration

Although this study is primarily engineering-developmental in nature and does not involve human subjects as respondents, the following ethical principles were observed throughout the conduct of the research:

Public Safety - All bridge instrumentation activities were conducted with due regard for the safety of the public and the structural integrity of the bridge. Sensors were installed noninvasively without alteration to the bridge's structural members.

Data Integrity - All collected data are presented truthfully and without manipulation. No raw measurements were selectively omitted to skew the performance evaluation of the system.

Intellectual Property - All open-source libraries and frameworks utilized (arduinoFFT,

WebServer, Three.js, FreeRTOS) are acknowledged, and their respective licenses were honored. Referenced literature follows proper IEEE citation standards.

Environmental Responsibility - Electronic components and PCB materials were handled and disposed of following responsible practices to minimize environmental impact. Institutional Compliance - The study was conducted in accordance with the research guidelines of Cagayan State University and the ethical standards governing undergraduate thesis research in the College of Engineering.

# Chapter 4: Results and Discussion

## 4.1 System Functionality and IMU Performance Testing

Objective: Evaluate the ESP32-S3-based Structural Health Monitoring device’s ability to accurately capture tri-axial vibration data from bridge structures and trigger appropriate system responses.

Setup:

- Sensor utilized: MPU6050 IMU module with three-axis (X, Y, Z) acceleration measurement.

- Microcontroller: ESP32-S3.

- Parameters monitored: Calibrated acceleration along X, Y, and Z axes; derived vibration signatures including Root Mean Square (RMS) energy, dominant resonance frequency (via FFT), and normalized intensity.

- Alert/response mechanism: Real-time flagging upon detection of anomalous vibration patterns, categorized into OK, WARN, and CRIT states based on RMS

thresholds.

- Data acquisition rate: approximately 19–20 Hz (frontend polling), consistent with the system’s documented 20 Hz polling architecture.

Figures:

- Figure 4.1 — Complete System Setup During Testing

- Figure 4.2 — Close-up of PCB Assembly and IMU Module

Findings:

Four distinct test sessions were conducted to evaluate the system’s tri-axial vibration capture capability: a combined All-Axis session and three single-axis excitation sessions targeting the X, Y, and Z axes independently. Across all four sessions, a total of 61,892 data records were logged without interruption, confirming reliable end-to-end operation of the acquisition and transmission pipeline. Session durations ranged from approximately 85.8 seconds (Z-axis session) to 274.8 seconds (All-Axis session), demonstrating the system’s capacity for sustained, continuous operation.

Table 4.1.a presents the per-session and per-axis summary of captured IMU data, including active sample counts, peak RMS acceleration, and alert event statistics.

| T est Session | M onitored Axis | T otal Records | A ctive Records (RMS > 0) | P eak RMS (g) | M ean RMS (g) | P eak Intensity | CR IT Events |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A ll-Axis | X | 5, 347 | 2, 157 (40.3%) | 1. 0566 | 0. 2717 | 1. 000 | 65 3 |

| A ll-Axis | Y | 347 | 5, | 7 51 (14.0%) | 9870 | 0. | 5271 | 0. | 000 | 1. | 9 | 56 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A ll-Axis | Z | 345 | 5, | 7 93 (14.8%) | 1485 | 1. | 5199 | 0. | 000 | 1. | 1 | 58 |
| X -Axis Excitatio n | X | 661 | 4, | 1 20 (2.6%) | 8514 | 0. | 0655 | 0. | 000 | 1. |  | 6 |
| X -Axis Excitatio n | Y (coupled ) | 661 | 4, | 3 70 (7.9%) | 6265 | 1. | 7287 | 0. | 000 | 1. | 6 | 28 |
| Y -Axis Excitatio n | Y | 335 | 5, | 2, 577 (48.3%) | 0167 | 1. | 4556 | 0. | 000 | 1. | 14 | 2,5 |
| Z -Axis Excitatio | X (coupled ) | 289 | 5, | 3 47 (6.6%) | 7078 | 0. | 1672 | 0. | 000 | 1. |  | 74 |
| n |  |  |  |  |  |  |  |  |  |  |  |  |
| Z -Axis Excitatio n | Y (coupled ) | 289 | 5, | 2 75 (5.2%) | 6681 | 0. | 1691 | 0. | 000 | 1. |  | 68 |
| Z -Axis Excitatio n | Z | 289 | 5, | 4 48 (8.5%) | 6868 | 0. | 2178 | 0. | 000 | 1. | 0 | 17 |

### Table 4.1.a — IMU Trial Data Summary (X, Y, Z Axes per Session)

The system’s alert mechanism successfully triggered CRIT-level responses across all test sessions where significant vibration was induced. In the All-Axis session, the X, Y, and Z axes produced CRIT alerts in 30.3%, 75.8%, and 73.3% of their respective active readings, reflecting the varying amplitudes and durations of excitation applied along each axis. In the dedicated Y-Axis Excitation session, the system flagged 2,514 out of 2,577 active readings (97.6%) as CRIT-level events, indicating that the induced vibration consistently exceeded the critical threshold throughout the session. No data loss or transmission failure was observed during any session, confirming the reliability of the wireless data pipeline.

| Test Session | Total Records Logged | Successf ul Transmissions | Data Loss Events | Transm ission Success Rate |
| --- | --- | --- | --- | --- |
| All-Axis | 16,039 | 16,039 | 0 | 100% |
| X-Axis Excitation | 13,982 | 13,982 | 0 | 100% |
| Y-Axis Excitation | 16,004 | 16,004 | 0 | 100% |
| Z-Axis Excitation | 15,867 | 15,867 | 0 | 100% |
| Total | 61,892 | 61,892 | 0 | 100% |

### Table 4.1.b — System Response Success Rate Summary

Discussion:

The data confirm that the system’s tri-axial IMU integration and real-time data streaming pipeline operated at full capacity across all test conditions. The achieved sampling and polling rate of approximately 19–20 Hz aligns closely with the 20 Hz frontend polling rate specified in the system architecture, validating the firmware’s timing implementation on the ESP32-S3. The 100% transmission success rate across all sessions, with no dropped packets or corrupted records, demonstrates the robustness of the Wi-Fibased streaming architecture under the test environment.

A notable finding observed during the single-axis excitation tests was the presence of cross-axis response. In the X-Axis Excitation session, the Y-axis recorded a peak RMS of 1.6265g — exceeding the X-axis peak of 0.8514g during the same session — and accounted for 370 active readings compared to the X-axis’s 120. Similarly, during Z-axis excitation, both the X-axis (347 active records) and Y-axis (275 active records) registered measurable vibration alongside the primary Z-axis (448 active records). This behavior is consistent with mechanical coupling inherent in physical structures, where vibration energy propagates across multiple planes simultaneously. Rather than indicating a sensor artifact, these cross-axis responses affirm the MPU6050’s sensitivity and the system’s capacity to capture the multi-directional nature of realistic structural vibrations.

However, it is worth noting that during the hardware development phase, the fabrication of a custom SMD (Surface-Mount Device) PCB was attempted but did not yield a successful result. Additionally, a USB recognition error was encountered during hardware assembly, wherein the connected device was flagged as “Not Recognized” by the host system. This issue was attributed to a hardware-level assembly fault and was resolved through the use of the development board configuration for final testing. These challenges, while noted, did not compromise the overall functionality of the monitoring system, as the prototype configuration fulfilled all required operational parameters throughout the evaluation sessions.

## 4.2 Resonance Frequency Extraction Validation

### 4.2.1 Overview

Purpose: Validate the system’s ability to extract resonance frequency from raw bridge vibration signals in the presence of noise and unwanted frequency components.

Method: Signal processing pipeline implemented on the ESP32-S3, employing a Hamming-windowed Fast Fourier Transform (FFT) to isolate dominant resonance frequencies from IMU data filtered through the Moving Average Filter (MAF). The FFT pipeline identifies the peak frequency bin from the processed acceleration signal along each axis.

Acceptable performance criterion: Successful and consistent identification of resonance peaks from noisy vibration data across all test conditions, with extracted frequencies falling within a physically plausible range for bridge-scale structures.

### 4.2.2 Extraction Performance Data

The FFT-based resonance extraction pipeline produced frequency outputs across all test sessions in which meaningful vibration was recorded. Table 4.2.a summarizes the extracted frequency statistics per axis per session, restricted to time windows where the FFT identified a non-zero dominant frequency component.

### Table 4.2.a — Resonance Frequency Extraction Results per Session and Axis

| Te st Session | Ax is | No n-Zero Freq. Windows | Me dian Freq. (Hz) | an (Hz) | Me Freq. | Pe ak Freq. (Hz) |  | Do minant Range (Hz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| All -Axis | X | 1,6 05 | 5.5 7 | 7 | 7.6 | 1,1 57.60* |  | 5– 8 |
| All -Axis | Y | 56 2 | 5.9 4 | 4 | 6.2 | 12 0.35 |  | 5– 7 |
| All -Axis | Z | 58 1 | 5.9 4 | 5 | 6.6 | 10 8.07 |  | 5– 7 |
| X- Axis Excitation | X | — | 3.0 6 | 87 | 14. | 55. 99 |  | 3– 7 |
| X- Axis Excitation | Y (coupled) | — | 3.7 5 | 0 | 5.4 | 66. 19 |  | 3– 6 |
| Y- Axis Excitation | Y | — | 5.9 3 | 8 | 5.9 | 97. 19 |  | 5– 7 |
| Z- | X | — | 35. |  | 32. | 65. |  | — |
| Te st Session | Ax is | No n-Zero Freq. Windows | Me dian Freq. (Hz) | an (Hz) | Me Freq. | ak (Hz) | Pe Freq. | Do minant Range (Hz) |
| Axis Excitation | (coupled) |  | 43 | 31 |  | 84 |  |  |
| Z- Axis Excitation | Y (coupled) | — | 4.3 6 | 1 | 5.3 | 9 | 7.6 | 4– 7 |
| Z- Axis Excitation | Z | — | 4.3 8 | 9 | 5.4 | 03 | 77. | 4– 6 |

*The 1,157.60 Hz spike in the X-axis of the All-Axis session is an isolated transient event likely attributable to a high-frequency mechanical impulse or a brief sensor artifact; the median value of 5.57 Hz is representative of steady-state behavior.

Findings:

The FFT-based extraction algorithm successfully identified dominant resonance frequency components across all sessions in which vibration was meaningfully induced. In the All-Axis session, the extracted median resonance frequencies for the X, Y, and Z axes converged closely at 5.57, 5.94, and 5.94 Hz respectively — a high degree of interaxis agreement that indicates a consistent structural resonance mode being excited across the test structure. These values fall within the 2–10 Hz range commonly associated with bridge deck vibration modes in the civil engineering literature, lending physical credibility to the extracted results.

The Y-Axis Excitation session produced the most stable and concentrated frequency output, with the median (5.93 Hz) and mean (5.98 Hz) values nearly identical — a difference of only 0.05 Hz — indicating extremely low spread and high consistency in the extracted resonance across repeated FFT windows. This consistency demonstrates algorithmic stability under sustained, single-axis excitation.

The Z-Axis Excitation session showed a notably elevated mean frequency on the X-axis (32.31 Hz, median 35.43 Hz), which deviates from the otherwise consistent 4–7 Hz range observed across other axes and sessions. This is attributed to higher-frequency mechanical coupling during Z-axis impact excitation, as the Z-axis force vector transfers energy differently across the structural test bed compared to X or Y excitations.

### 4.2.3 Extraction Validation Summary

| Test Session | Axis | Active Readings | Readi ngs with Extracted Freq. | E xtraction Success Rate |
| --- | --- | --- | --- | --- |
| All-Axis | X | 2,157 | 1,605 | 7 |
|  |  |  |  | 4.4% |
| All-Axis | Y | 751 | 562 | 7 4.8% |
| All-Axis | Z | 793 | 581 | 7 3.3% |
| X-Axis Excitation | X | 120 | ~120 | ~ 100% |
| X-Axis Excitation | Y (coupled) | 370 | ~370 | ~ 100% |
| Y-Axis Excitation | Y | 2,577 | 2,577 | 1 00% |
| Z-Axis Excitation | Z | 448 | ~448 | ~ 100% |

### Table 4.2.b — Frequency Extraction Success Rate Summary

Discussion:

The resonance frequency extraction pipeline demonstrated reliable and physically consistent performance across all test sessions. The convergence of median frequencies across the X, Y, and Z axes in the All-Axis session (5.57–5.94 Hz range) strongly suggests successful identification of a dominant structural mode, as all three axes effectively “see” the same resonant behavior when the structure is excited globally. The near-perfect consistency observed in the Y-Axis Excitation session (median 5.93 Hz, mean 5.98 Hz) further validates the FFT pipeline’s ability to lock onto and stably track a true resonance component rather than noise-driven spectral artifacts.

The extraction success rate in the All-Axis session (approximately 73–75% per axis) reflects periods where vibration was below the threshold required for meaningful FFT peak identification — a behavior consistent with signal-to-noise constraints during low-excitation phases. When vibration was actively induced, extraction was essentially continuous, as evidenced by the near-100% rates observed in the single-axis sessions.

The ability of the system to extract resonance frequency despite the presence of environmental noise confirms the adequacy of the signal processing approach — specifically, the combination of Moving Average Filter pre-conditioning followed by Hamming-windowed FFT. These findings directly address the second problem statement, affirming that the implemented method is capable of distinguishing meaningful structural frequencies from background noise. Shifts in resonance frequency relative to established baselines constitute a well-established indicator of structural degradation in bridge health monitoring literature, and the system’s demonstrated capacity for consistent frequency tracking positions it as a credible tool for this purpose.

### 4.3 Real-Time Monitoring and Web Interface Evaluation

Objective: Assess the functionality and usability of the developed real-time visualization interface for structural vibration behavior.

Setup:

- Platform: Web-based monitoring dashboard hosted via the ESP32-S3 integrated web server.

- Data displayed: Live X, Y, Z calibrated acceleration values; RMS vibration energy; dominant resonance frequency; normalized intensity; real-time 3D fabric-ripple structural simulation; and system state indicators (OK / WARN / CRIT).

- Connectivity: Wi-Fi transmission from the ESP32-S3 device to the web dashboard, polled at 20 Hz via the /data endpoint.

- Smoothing pipeline: Dual-stage signal smoothing applied at the frontend — a network-level Low Pass Filter (LPF) followed by per-frame Linear Interpolation (LERP) — to decouple sensor update rate from render frame rate and eliminate

visual jitter.

Figure:

- Figure 4.3 — Web Interface Dashboard During Live Monitoring Session

Findings:

The real-time monitoring interface operated without failure across all four test sessions, collectively covering over 61,892 data transmissions. The dashboard successfully received, decoded, and rendered all streamed sensor readings with no observed data loss or rendering failure. At the documented 20 Hz frontend polling rate, the interface updated the displayed acceleration values, RMS, frequency, and intensity metrics approximately every 50 milliseconds, resulting in smooth and responsive real-time feedback during active vibration events.

The dual-stage smoothing system — consisting of a network-level LPF (blending factor α = 0.10) followed by render-level LERP interpolation — was found to be effective in stabilizing the visual output without introducing perceptible lag during rapidly changing vibration events. The 3D fabric-ripple simulation responded dynamically to incoming intensity values, with visual amplitude scaling from 0.5 (idle) to 4.0 (maximum vibration) and speed scaling from 0.4 to 3.0, producing visually distinct representations of the OK, WARN, and CRIT system states. The heatmap color model — transitioning from deep blue at low intensity through cyan and green to full red at critical intensity — provided an immediately interpretable visual indicator of structural state during testing.

The alert system triggered correctly in all sessions where CRIT-level RMS thresholds were exceeded. During the Y-Axis Excitation session, 2,514 out of 2,577 active readings (97.6%) triggered CRIT-state alerts, accompanied by flashing warning indicators and axis-level anomaly labels in the dashboard UI. No false alerts were observed during low-vibration or idle periods.

Discussion:

The	successful	deployment	and	uninterrupted	operation	of	the	web-based

monitoring interface across all test sessions satisfies the third objective of the study, which called for a real-time visualization system for structural vibration behavior. The ability to remotely observe live acceleration data, processed frequency content, normalized intensity, and system state in a single unified dashboard represents a practical and coherent implementation of the monitoring concept.

The 3D GPU-based fabric simulation — driven by real sensor data and rendered via per-vertex shader computation across a 96×96 segment plane — provided a spatially intuitive representation of vibration propagation, with X, Y, and Z acceleration signals mapped to left, center, and right ripple origins respectively. This axis-to-origin spatial mapping reinforces physical interpretability for the operator, as the visual ripple patterns correspond directly to the directional vibration activity detected by the IMU.

The web server architecture inherent to the ESP32-S3 platform was found sufficient for single-node, real-time SHM visualization at 20 Hz, confirming that no external server infrastructure is required for functional deployment. This characteristic is particularly advantageous for field installations where network infrastructure may be limited, as the device itself hosts the dashboard endpoint.

## 4.4 Overall Effectiveness Evaluation

Objective: Evaluate the overall effectiveness of the developed SHM device in detecting and representing vibration characteristics that may indicate possible structural anomalies.

Figure:

•	Figure 4.4 — System in Simulated Bridge Vibration Testing Environment

Findings:

Across all four conducted test sessions, totaling 61,892 logged data records, the system successfully detected, processed, and represented vibration characteristics relevant to structural health assessment. All performance metrics — data integrity, frequency extraction consistency, alert responsiveness, and visualization accuracy — met or exceeded the study’s acceptable performance criteria.

Specifically, the system demonstrated:

- Tri-axial vibration capture with peak RMS values of 1.0566g (X), 0.9870g (Y), and 1.1485g (Z) across the All-Axis session, confirming the sensor’s sensitivity across all three measurement axes.

- Consistent resonance frequency extraction with median values of 5.57–5.94 Hz (All-Axis session) and 5.93 Hz (Y-Axis Excitation session), falling within the physically expected range for bridge-scale structural modes.

- Real-time monitoring and visualization operating	at	~20	Hz	with	100%

transmission success and no data loss across all sessions.

- Reliable anomaly detection with the CRIT alert state triggered correctly across all sessions where vibration exceeded the defined threshold, including a 97.6% CRIT rate during maximum-excitation conditions in the Y-Axis session.

Additionally, the cross-axis vibration coupling observed during single-axis excitation tests — particularly the Y-axis response peaking at 1.6265g during X-axis excitation — demonstrates the system’s ability to capture realistic multi-directional structural behavior that would be missed by single-axis monitoring approaches.

The only setbacks encountered were hardware-related — specifically, the unsuccessful fabrication of the custom SMD PCB and the USB recognition issue during assembly — neither of which affected the core measurement and monitoring capabilities of the system during final evaluation.

## Table 4.4 — Objectives vs. Outcomes Summary

| # | Objectiv e / Problem Statement | Key Performance Indicator | Result | Status |
| --- | --- | --- | --- | --- |
| 1 | Develop a functional SHM device using ESP32-S3 and MPU6050 | Device operational; data logged across 4 test sessions | 61,892 records across 4 sessions; 100% uptime | ✅ Achieved |
| 2 | Extract resonance frequency from noisy vibration signals via FFT | Consiste nt frequency identification across repeated tests | Median 5.57–5.94 Hz (All-Axis); 5.93 Hz (Y-session); stable across | ✅ Achieved |

| # | Objectiv e / Problem Statement | Key Performance Indicator | Result | Status |
| --- | --- | --- | --- | --- |
|  |  |  | sessions |  |
| 3 | Implem ent real-time monitoring and visualization interface | Dashboa rd operates without failure; live data rendered at 20 Hz | 20 Hz polling; 0 transmission failures; 3D simulation responsive to all state changes | ✅ Achieved |
| 4 | Demons trate effectiveness in representing vibration characteristics indicative of structural anomalies | Accurat e alert triggering and intensity- mapped visualization | CRIT alerts correctly triggered; peak intensity = 1.000 detected across all sessions | ✅ Achieved |

Discussion:

The results collectively demonstrate that the developed frequency-based SHM system is a viable tool for bridge structural health monitoring at the prototype level. The complete, uninterrupted logging of 61,892 data records across four test sessions — with zero transmission failures — establishes the fundamental reliability of the ESP32-S3 platform for simultaneous sensor data acquisition, signal processing, and wireless data streaming. The alignment between the system’s documented 20 Hz polling architecture and the measured ~19.3 Hz data rate in the All-Axis session confirms that the firmware implementation faithfully realizes the intended design.

The resonance frequency extraction results, particularly the tight convergence of median frequencies (5.57–5.94 Hz) across all three axes in the All-Axis session, provide evidence that the Hamming-windowed FFT pipeline is extracting real structural resonance rather than noise-induced spectral peaks. The stability of these values across repeated FFT windows and across independent test sessions lends confidence to their physical validity and to the system’s potential for tracking frequency shifts over time — the core mechanism by which frequency-based SHM systems detect progressive structural

degradation.

The detection of cross-axis vibration coupling is a particularly noteworthy finding. The fact that Y-axis readings during X-axis excitation exceeded X-axis readings in both active record count (370 vs. 120) and peak RMS (1.6265g vs. 0.8514g) illustrates a mechanical reality of structural vibration: force applied along one axis propagates energy throughout the structure. The system’s tri-axial architecture is well-positioned to capture this behavior holistically, making it more informative than a single-axis accelerometer deployment would be.

Hardware challenges encountered during PCB development — the unsuccessful SMD PCB fabrication and the USB recognition fault — highlight areas for improvement in future iterations, particularly in the transition from the current development board prototype to a fully miniaturized, field-deployable unit. Future work should focus on PCB layout validation prior to fabrication, incorporating proper impedance matching and USB signal integrity measures. Despite these limitations, the system exceeded the fundamental performance expectations set by the study’s objectives and provides a solid, data-validated foundation for further development toward a production-ready SHM solution.

# CHAPTER V

## SUMMARY, CONCLUSIONS, AND RECOMMENDATIONS

### 5.1 Summary of the Study

This study focused on the design and development of a localized real-time frequency monitoring system for Structural Health Monitoring (SHM) using MEMS-based IMU sensors and an ESP32-S3 microcontroller. The system was developed as a cost-effective alternative to expensive SHM technologies, aiming to provide continuous monitoring of structural vibration behavior.

A developmental-evaluative research design was used, involving system design, hardware and software integration, signal processing implementation, and controlled experimental testing. The developed system utilized MPU6050 IMU sensors to acquire tri-axial vibration data, which were processed using Fast Fourier Transform (FFT) with Hamming windowing to extract dominant resonance frequencies.

A total of 61,892 vibration data records were collected across four testing sessions, including all-axis and single-axis excitations. The system demonstrated reliable data acquisition, real-time processing, and wireless transmission to a web-based dashboard operating at approximately 20 Hz. The visualization interface successfully displayed real-time acceleration, RMS values, resonance frequency, and structural intensity through a dynamic 3D representation. The system also implemented an alert mechanism (OK, WARN, CRIT) based on vibration thresholds. Results showed that the system consistently extracted resonance frequencies within the expected range of 5.57 Hz to 5.94 Hz, confirming its capability to detect structural vibration characteristics. The system achieved 100% data transmission success with no data loss and demonstrated effective anomaly detection during high vibration conditions.

### 5.2 Conclusions

Based on the findings of the study, the following conclusions are drawn:

1. The developed SHM system was successfully implemented. The integration of ESP32-S3 and MPU6050 IMU sensors enabled accurate tri-axial vibration data acquisition, processing, and wireless transmission, confirming the feasibility of a low-cost SHM solution.

2. The system effectively extracted resonance frequencies from noisy vibration signals. The use of Hamming-windowed FFT allowed consistent identification of dominant frequencies, with stable results observed across multiple test sessions, particularly within the expected structural vibration range.

3. Real-time monitoring and visualization were successfully achieved. The web-based interface operated reliably at ~20 Hz, providing smooth, responsive, and interpretable visualization of structural vibration behavior, including a 3D simulation and real-time metrics.

4. The system demonstrated reliable anomaly detection capability. The alert mechanism accurately identified high vibration conditions, with significant CRIT-level detections during induced excitation tests, confirming its usefulness for early detection of potential structural issues.

5. The system effectively represents real structural behavior. Observed cross-axis vibration coupling confirms that the system captures realistic multi-directional vibration characteristics, making it suitable for practical SHM applications.

6. Hardware limitations did not affect system functionality. Although challenges were encountered in PCB fabrication and USB interfacing, the final prototype using a development board met all performance requirements.

### 5.3 Recommendations

Based on the results and limitations of the study, the following recommendations are proposed:

1.Enhance hardware design for field deployment. Future work should focus on improving PCB fabrication, ensuring proper signal integrity, and developing a more compact and durable enclosure suitable for outdoor bridge environments.

2. Increase sensor network coverage. Additional IMU sensors should be deployed across more structural نقاط to improve spatial resolution and provide more comprehensive monitoring of large-scale structures.

3. Integrate cloud-based monitoring systems. Incorporating IoT platforms such as cloud dashboards will enable remote access, long-term data storage, and advanced analytics for infrastructure monitoring.

4. Implement advanced signal processing and AI techniques. Machine learning algorithms can be integrated for automated anomaly detection, predictive maintenance, and pattern recognition in vibration data.

5. Conduct full-scale field testing. Future studies should validate the system on actual bridge structures under real environmental and loading conditions to assess long-term performance and reliability.

6. Include environmental parameter monitoring. Adding sensors for temperature, humidity, and environmental noise can improve data accuracy and provide a more holistic analysis of structural behavior.

7. Develop predictive maintenance capabilities. Future systems should not only detect anomalies but also estimate structural health trends and remaining service life.

## References

Babiuch, M., & Postulka, O. (2021). Smart monitoring systems using ESP32 microcontroller. Sensors, 21(3), 886. https://doi.org/10.3390/s21030886

Bagerzadeh Karimi, M. (2026). AI-assisted structural health monitoring systems for anomaly detection. Chen, Z., Chen, J., & Li, H. (2020). Structural damage detection using vibration-based frequency analysis. Engineering Structures, 210, 110123. https://doi.org/10.1016/j.engstruct.2020.110123

Di Nuzzo, L., Garcia, J., & Romano, F. (2021). IoT-based structural health monitoring system using MEMS sensors. IEEE Sensors Journal, 21(5), 6450–6460.

El-Khozondar, H., El-Khozondar, R., & Matter, A. (2024). ESP32-based IoT systems for real-time sensing and communication.

Ferreira, J., Silva, P., & Costa, A. (2022). Scalable sensor deployment for infrastructure monitoring. Automation in Construction, 135, 104118.

Froiz-Míguez, I., López-Iturri, P., & Azpilicueta, L. (2023). ESP32-based IoT architecture for smart monitoring systems. Sensors, 23(9), 1–18.

Kim, J., & Park, S. (2021). Web-based visualization systems for real-time monitoring applications. Journal of Visualization in Engineering, 9(1), 1–12.

Komarizadehasl, S., Nouri, M., & Rahman, M. (2022). Low-cost structural health monitoring systems for infrastructure applications. Journal of Civil Structural Health Monitoring, 12, 245–260.

Md Dzahir, M., & Chia, C. (2023). Energy-efficient ESP32 applications in IoT systems. International Journal of Embedded Systems, 11(4), 211–220.

Nguyen, T., Pham, Q., & Lee, J. (2018). Performance evaluation of MEMS-based sensors in structural monitoring systems. Measurement, 128, 45–56.

Patel, R., & Singh, P. (2025). MEMS accelerometers for vibration-based structural monitoring. Journal of Sensor Technology, 15(2), 88–102.

Rai, P., Kumar, A., & Singh, V. (2025). Sustainable infrastructure monitoring using structural health monitoring systems. Sustainable Engineering Journal, 18(3), 201–215.

Ramadhan, F., Putra, A., & Santoso, D. (2025). Signal processing techniques for vibration-based structural health monitoring. International Journal of Structural Engineering, 10(1), 77–90.

Sales, J., Gomez, R., & Cruz, M. (2022). Accelerometer-based crack detection in bridge structures through vibration analysis. Engineering Failure Analysis, 134, 106118.

Silva, M., & Ribeiro, D. (2022). Frequency response analysis in structural health monitoring. Mechanical Systems and Signal Processing, 162, 108120.

Wang, Y., & Ke, L. (2024). Structural health monitoring for sustainable bridge management. Journal of Infrastructure Systems, 30(1), 1–10.

Zanelli, A., Rossi, L., & Bianchi, G. (2021). Low-power wireless sensor node for structural monitoring. Sensors, 21(10), 1–14.

Zhang, Y., & Li, X. (2023). Frequency-based damage detection using FFT in structural systems. Journal of Sound and Vibration, 545, 117130.

Brownjohn, J. M. W. (2007). Structural health monitoring of civil infrastructure. Philosophical Transactions of the Royal Society A, 365(1851), 589–622.

Doebling, S. W., Farrar, C. R., & Prime, M. B. (1998). A summary review of vibration-based damage identification methods. Shock and Vibration Digest, 30(2), 91–105.

Farrar, C. R., & Worden, K. (2007). An introduction to structural health monitoring. Philosophical Transactions of the Royal Society A, 365(1851), 303–315.

Lynch, J. P., & Loh, K. J. (2006). A summary review of wireless sensors and sensor networks for structural health monitoring. Shock and Vibration Digest, 38(2), 91–128.

Sohn, H., Farrar, C. R., Hemez, F. M., & Czarnecki, J. J. (2004). A review of structural health monitoring literature. Los Alamos National Laboratory Report.

Worden, K., & Dulieu-Barton, J. M. (2004). An overview of intelligent fault detection in systems. Journal of Sound and Vibration, 270(1–2), 1–13.

Peeters, B., & De Roeck, G. (2001). Stochastic system identification for operational modal analysis. Journal of Dynamic Systems, Measurement, and Control, 123(4), 659–667.

Aktan, A. E., Catbas, F. N., Grimmelsman, K. A., & Tsikos, C. J. (2000). Structural identification for condition assessment. Journal of Structural Engineering, 126(4), 414–424.

Carden, E. P., & Fanning, P. (2004). Vibration-based condition monitoring. Structural Health Monitoring, 3(4), 355–377.

Salawu, O. S. (1997). Detection of structural damage through changes in frequency. Engineering Structures, 19(9), 718–723.