|  | Cagayan State University COLLEGE OF ENGINEERING AND ARCHITECTURE, CARIG CAMPUS COEA-SR01 |
| --- | --- |

Research Topic Pre-Registration Form

This form establishes a broad framework for the research topic that the student will elaborate on during the research proposal period, leading to the submission of a peer-reviewed thesis proposal.

THESIS TOPIC

| Thesis Title Development of a Localized Real-Time Frequency Monitoring System for Structural Health Monitoring Using MEMS IMUs and ESP32 | Program | ECE |
| --- | --- | --- |
|  | Major |  |

PROPONENT(S)

| Last Name, Given Name, Middle Initial | Signature |
| --- | --- |
| [RESEARCHER NAME] |  |

CONTACT INFORMATION

| Contact No/Name | Email Address |
| --- | --- |
| [RESEARCHER NAME] - [CONTACT NO.] | [EMAIL ADDRESS] |

BACKGROUND / DESCRIPTION OF TOPIC (including key references)

Please provide a general introduction of the topic; briefly review the current research in the field; highlight your own research experience in this field and any preliminary work that has been conducted on this topic.

Long-term bridge monitoring literature reports that temperature, wind, and traffic can affect measured dynamic response (Cross et al., 2013). Frequency-domain peak picking is distinct from full operational modal analysis; this study will report local spectral-frequency indicators and will not treat them as confirmed structural modes or damage diagnoses (Hasani & Freddi, 2023). The study proposes one fixed roadway-deck station at Buntun Bridge, Tuguegarao City, Cagayan, using one PCB with an ESP32-S3 and three co-located triaxial MPU-6050 MEMS IMUs. The system will sequentially acquire timestamped X-, Y-, and Z-axis acceleration records, derive dominant spectral-frequency indicators from 60-second windows, and display session summaries with vehicle counts recorded by a separate observer. Traffic-count groups will be defined locally during a pilot. The work focuses on developing and evaluating the acquisition, processing, and visualization workflow; it does not assess bridge safety, diagnose damage, or estimate bridge-wide modal shapes. No field results are yet available.

(you may attach additional pages if this space is lacking)

| KEYWORDS | Buntun Bridge; roadway-deck vibration; dominant spectral frequency; FFT; MEMS IMU |
| --- | --- |

TOPIC SIGNIFICANCE

Please provide information on how this topic leads to significant advances in the field of study.

The study will document a prototype and a proposed field procedure for recording local roadway-deck vibration-frequency indicators alongside observed traffic counts at one Buntun Bridge station. Its outputs may support future site-specific monitoring research. The study does not establish a structural-health diagnosis, a safety threshold, or a replacement for bridge inspection.

(you may attach additional pages if this space is lacking)

I hereby certify that the proponent(s) have submitted to me their synthesized review of literature as part of the requirements for the submission of the accomplished Research Topic Pre-Registration Form.

[SUBJECT ADVISER NAME]

SUBJECT ADVISER

Signature over Printed Name/Date